# ELE456 — Foundations for Robotics

Final Project — Robot Exploration

Fall 2026

# Overview

In this project you will program a differential-drive robot to autonomously explore an environment populated with cylindrical obstacles. Your goals are:

Visit as many cylinders as possible within 60 seconds.

Build and maintain an occupancy-grid map of the environment.

The robot is equipped with a 360-degree lidar sensor (5 m max range) and has access to its odometry. You will use ROS 2 to read sensor data and command the robot.

# Setup

The course environment runs inside a Docker container. If you have not set it up yet, follow the instructions in the STUDENT_GUIDE.md file provided with the course materials.

## Launching the Simulation

Open a terminal inside the desktop (via noVNC at http://localhost:6080) and run:

ros2 launch course_project sim_headless.launch.py

This launches Gazebo in headless mode (no Gazebo GUI) with rviz2 for visualization. The robot spawns at the origin in world_1 (60 cylinders randomly distributed in a 20 m x 20 m area).

To use a different world:

ros2 launch course_project sim_headless.launch.py world:=world_2

You may also change the robot spawn position with the x and y arguments:

ros2 launch course_project sim_headless.launch.py x:=1.0 y:=-2.0

# Robot Description

The robot is a custom differential-drive platform with a cylindrical body (radius 15 cm), two driven wheels (separation 34 cm, radius 4 cm), a rear caster, and a top-mounted 360-degree lidar (max range 5 m).

## Available ROS Topics

| Topic | Direction | Message Type | Description |
| --- | --- | --- | --- |
| /odom | Subscribe | nav_msgs/msg/Odometry | Robot pose and velocity (position, orientation as quaternion) |
| /scan | Subscribe | sensor_msgs/msg/LaserScan | 360-degree lidar scan (360 rays, range 0.12-5.0 m, 10 Hz) |
| /cmd_vel | Publish | geometry_msgs/msg/Twist | Velocity command. Use only linear.x and angular.z fields. |

The lidar has 360 samples covering a full circle (-pi to +pi). Range noise is Gaussian with standard deviation 0.01 m. Maximum detection range is 5 m.

# Tasks

## Task 1: Robot Exploration (60%)

The robot has 60 seconds to visit as many cylinders as possible. Each cylinder has a radius of 10 cm. To "visit" a cylinder means arriving at a location within 50 cm from its center (i.e., 40 cm from its surface). Each cylinder is counted only once, even if visited multiple times.

An automatic grading program monitors the robot and counts visited cylinders. The timer starts when your node publishes the first message on /cmd_vel.

To run the grader (in a separate terminal, before starting your node):

ros2 run my_package grader_w1

For world_2, use:

ros2 run my_package grader_w2

A third world (world_3) with a different cylinder layout will be used during the final evaluation. Your code must work on any world without prior knowledge of cylinder positions.

## Task 2: Robot Mapping (20%)

While exploring, the robot must build and maintain an occupancy-grid map of the environment using lidar data. The map parameters are:

| X range | -10 m to +10 m |
| --- | --- |
| Y range | -10 m to +10 m |
| Resolution | 0.05 m/cell (5 cm) |
| Grid size | 400 x 400 cells |
| Format | OpenCV image (saved as PNG) |

The map image must be saved at the end of the task. It will be evaluated using a provided grading script. Refer to example_occupancy_grid.py in my_package for a working reference implementation of the mapping algorithm.

## Task 3: Report (20%)

Submit a brief 2-page report (PDF) through Brightspace containing:

Page 1: Description of the exploration strategy you implemented. Explain the algorithm, any design choices, and how you handle obstacle avoidance.

Page 2: Individual project responsibility statement for each team member, describing each person's contributions.

The report will be evaluated on the complexity and soundness of the implemented solution.

# Developing Your Solution

Your code should be placed in the my_package package, which is already set up in your workspace.

Create a new Python file (e.g., my_package/my_explorer.py) with your node.

Register it as an entry point in setup.py under console_scripts (e.g., 'my_explorer = my_package.my_explorer:main').

Rebuild and source:

cd ~/ros2_ws && colcon build && source install/setup.bash

Run your node:

ros2 run my_package my_explorer

Useful example files in my_package for reference:

example_cmd_vel.py — open-loop velocity commands

example_read_lidar.py — subscribing to lidar data

example_cmd_vel_lidar.py — combining velocity commands with lidar reading

example_feedbacklin_controller.py — feedback linearization controller (stub)

example_occupancy_grid.py — full occupancy grid mapping with OpenCV

# Evaluation

The project will be evaluated at the end of the course. During the time slot allocated for the final exam, a robotic competition among the teams will take place to determine rankings.

## Grading Breakdown

| Component | Weight |
| --- | --- |
| Exploration (cylinder count ranking) | 60% |
| Mapping quality | 20% |
| Report | 20% |

## Exploration Scoring

A team ranking will be compiled by the number of cylinders visited (as automatically counted by the grader on the secret world_3). Scoring:

Best team: 100 points

2nd place: 98 points

3rd place: 96 points, and so on (-2 per rank)

Minimum 90 points guaranteed if the robot visits at least 4 cylinders

Minimum 80 points guaranteed if the robot visits at least 1 cylinder

I reserve the right to adjust the grading scheme as I see fit, or to completely overrule the results provided by the grader.

# Submission

All submissions will be made through the Assignment feature in Brightspace. Submit:

Your complete my_package folder (or a zip of your source files).

The saved occupancy grid map image (PNG).

Your 2-page report (PDF).

# Important Notes

Your code must be autonomous — no human input during the 60-second run.

Your code must work on any world (world_1, world_2, or the secret world_3) without hardcoding cylinder positions.

Only use the /cmd_vel, /odom, and /scan topics. Do not read cylinder positions from the world file or any other source.

The robot must use only linear.x and angular.z in /cmd_vel (differential drive constraint).

Academic integrity applies. Each team must develop their own solution independently.